Application: Text Messaging

- Addressing:

  - 4 bits of addresses

  - 0xF is broadcast address

  - 8 bit transmit: 4 (src) and 4 (dst)

- Message structure:

  - Sync: 31 bits

  - DST: 4 bits

  - SRC: 4 bits

  - Sequence number: 8 bits

  - Payload: 160 bytes

MAC:

- Token passing “Ping Pong”

- Pass implicit with the message

- ACK:

  - contains receipt of message and whether receiver wants token

  - Structure:

    - DST: 4 bits

    - SRC: 4 bits

    - Packet or not: 1 bit

    - Want token or not: 1 bit

    - Sequence number: 8 bits

    - CRC: 8 bits

      - Poly: 100000111

- Possible cases:

- B receives, A receives ACK

- B receives, A doesn't get ACK

- B receives, CRC bad, sends NACK

- B doesn't receive

Physical:

- 31 bit PN sequence

- (8,4,3) Hamming code

- Scrambler/Descrambler – Provided

- BPSK modulation